4MP VF Bullet Network Cameras

M-49-V-V2

User Manual

Thank you for your purchase. If you have any questions, please do not hesitate to contact your dealer.

Disclaimer

|  | CAUTION! The default password is intended for your first login. For security, please set a strong password after your first login. A strong password shall include at least nine characters comprising digits, letters, and special characters. |
| --- | --- |

To the maximum extent permitted by applicable law, the product described, with its hardware, software, firmware, and documents, is provided on an “as is” basis.

Best effort has been made to verify the integrity and correctness of the contents in this manual, but no statement, information, or recommendation in this manual shall constitute formal guarantee of any kind, expressed or implied. We shall not be held responsible for any technical or typographical errors in this manual. The contents of this manual are subject to change without prior notice. Update will be added to the new version of this manual.

Use of this manual and the subsequent result shall be entirely on the user’s own responsibility. In no event shall we be reliable to you for any special, consequential, incidental, or indirect damages, including, among others, damages for loss of business profits, business interruption, or loss of data or documentation in connection with the use of this product.

Video and audio surveillance can be regulated by laws that vary from country to country. Check the law in your local region before using this product for surveillance purposes. We shall not be held responsible for any consequences resulting from illegal operations of the device.

The illustrations in this manual are for reference only and may vary depending on the version or model. The screenshots in this manual may have been customized to meet specific requirements and user preferences. As a result, some of the examples and functions featured may differ from those displayed on your monitor.

This manual is a guide for multiple product models and so it is not intended for any specific product.

Due to uncertainties such as physical environment, discrepancy may exist between the actual values and reference values provided in this manual. The ultimate right to interpretation resides in our company.

Environmental Protection

This product has been designed to comply with the requirements on environmental protection. For the proper storage, use and disposal of this product, national laws and regulations must be observed.

Safety Symbols

The symbols in the following table may be found in this manual. Carefully follow the instructions indicated by the symbols to avoid hazardous situations and use the product properly.

| Symbol | Description |
| --- | --- |
| WARNING! | Indicates a hazardous situation which, if not avoided, could result in bodily injury or death. |
| CAUTION! | Indicates a situation which, if not avoided, could result in damage, data loss or malfunction to product. |
| NOTE! | Indicates useful or supplemental information about the use of product. |

Contents

Safety Instructions	2

Environmental Protection	2

Safety Symbols	2

1 Login	5

1.1 Preparation	5

1.2 Login	5

1.2.1 Preparation	5

1.2.2 Logging in to the Web Interface	6

2 Live View	8

2.1 Live View	8

2.1.1 Pixel Calculation	10

2.1.2 Digital Zoom	11

2.1.3 Capture	11

2.2 General Parameters	12

3 Playback	12

3.1 Playback Toolbar	13

3.2 Search and Playback	14

3.3 Recording Download	14

4 Photo	17

5 Setup	18

5.1 Local Parameters	18

5.2 Network	19

5.2.1 Basic Config	19

5.2.2 Service Configuration	24

5.3 Video & Audio	29

5.3.1 Video	29

5.3.2 Snapshot	31

5.3.3 Audio	33

5.3.4 ROI	35

5.3.5 Media Stream	36

5.4 Image	37

5.4.1 Image	37

5.4.2 OSD	46

5.4.3 Privacy Mask	48

5.5 Smart	50

5.5.1 Alarm-triggered Actions	50

5.5.2 Arming Schedule	51

5.5.3 Cross Line Detection	52

5.5.4 Intrusion Detection	53

5.6 Alarm	56

5.6.1 Common Alarm	56

5.7 Storage	65

5.7.1 Memory Card	65

5.7.2 Network Disk	67

5.7.3 FTP	68

5.8 Security	69

5.8.1 User	69

5.8.2 HTTPS	72

5.8.3 Authentication	72

5.8.4 Registration Information	73

5.8.5 ARP Protection	73

5.8.6 IP Address Filtering	74

5.8.7 Access Policy	74

5.8.8 Watermark	76

5.9 System	77

5.9.1 Time	77

5.9.2 DST	78

5.9.3 Device Information	78

5.9.4 Maintenance	78

5.9.5 Logs	80

# Login

## Preparation

Refer to the camera’s quick guide to complete installation, and then connect the camera to power to start it up. You can log in to the camera’s web interface to perform management or maintenance operations.

Windows 7, 8 or later is installed on the PC. The following takes Windows 7 as an example.

Before accessing a network camera (also known as IP Camera or IPC) from a PC, you need to connect the network camera to the PC directly with a network cable or via a switch or router.

Use a Shielded Twisted Pair (STP) cable to connect the network interfaces of the network camera and the PC.

Use Shielded Twisted Pair (STP) cables to connect the network interfaces of the camera and the switch or router.

Check before login

The camera runs normally.

The PC has a network connection to the camera.

You have permission to operate the PC.

Google Chrome 105 or later is recommended on the PC.

A 32-bit or 64-bit Web browser is required if you are using a 64-bit operating system. It is recommended to use a monitor with the highest resolution for better display.

## Login

### Preparation

After you have completed the installation in accordance with the quick guide, connect the camera to power to start it. After the camera is started, you can access the camera from a PC client installed with a web browser or the client software Advidia Guard. Internet Explorer (IE) is a recommended web browser. For more information about Advidia Guard, refer to the Advidia Guard User Manual.

The following takes IE on a Microsoft Windows7 operating system as an example.

  - Check before login

The camera is operating correctly.

The network connection between the PC and the camera is normal.

The PC is installed with Internet Explorer 10.0 or higher.

(Optional) The resolution is set to 1440 x 900.

  - Add the IP address as a trusted site

|  | NOTE! The IP address 192.168.1.13in this example is the default IP address. Please replace it with the actual address of your camera if it has been changed. |
| --- | --- |

  - (Optional) Modify user access control settings

Before you access the camera, follow the steps to set User Account Control Settings to Never notify.

### Logging in to the Web Interface

The default static IP address of the camera is DHCP, you may use Advidia tools to scan for the camera’s IP address.

DHCP is enabled by default on the camera. If a DHCP server is deployed in the network, the camera may be assigned an IP address, and you need to use the assigned IP address to log in.

Follow the steps below to log in to the camera’s web interface:

- Open Chrome, enter the IP address of your camera in the address bar and press Enter.

At your first login, you need to follow the on-screen instructions to install a plug-in (close all browsers before installation), and then open the browser again to log in. To manually load the plug-in, type http://IP address/ActiveX/Setup.exe in the address bar and press Enter.

- Set whether to start live view automatically after login.

With Live View selected, live view will start automatically after login.

With Live View not selected, you need to start live view manually.

- Enter the username and password (admin/123456 by default) and click Login. To clear the Username and Password text boxes, click Reset.

After first login, the Change Password dialog box appears, in which you must set a strong password and enter your email address in case of password retrieval.

Set a strong password of 9 to 32 characters including all three elements: digits, letters, and special characters.

  - Enter your email address in case of password retrieval.

See User for more information.

If you forgot your password, click Forgot Password in the login page, then follow the on-screen instructions to reset your password.

|  | NOTE! For certain devices, you may need to choose whether to enable cloud service after your first login. |
| --- | --- |

# Live View

## Live View

After login, the Live View page appears, showing the live video from the camera. You may double-click the window to enter or exit full screen mode.

- Live view page of single-channel camera

|  | NOTE! Live view operations supported may vary with device model. |
| --- | --- |

| Live View toolbar |  |
| --- | --- |
| Item | Description |
|  | Set the image display ratio in the window. Scale: Displays 16:9 images. Stretch: Displays images according to the window size (stretch images to fit the window). Original: Displays images with original size. |
|  | Set the image display mode in the window. - Single Channel: Displays live video of a single channel. - Left/Right Split: Displays live video in left/right split mode. - Top/Bottom Split: Displays live video in top/bottom split mode. - Picture in Picture: Opens a floating live view window on top of the current window. NOTE! This function is only available on dual-channel cameras. |
|  | : Stop/start live view of the selected channel. : Start local recording. : Switch streams. |
|  | Select a live video stream according to your camera. |
|  | Set image parameters. |
| / | Start/stop live view. |
| / | Turn off/on sound. |
|  | Adjust the output volume for the media player on the PC. Range: 1 to 100. |
|  | Adjust the microphone volume on the PC during audio communication between the PC and the camera. Range: 1 to 100. |
|  | Frame rate/bit rate/resolution/packet loss rate. |
| / | Enable/disable pixel calculation. |
|  | Take a snapshot from the displayed live video. NOTE! See Local Parameters for the path of the saved snapshots. |
| / | Start/stop local recording. NOTE! - See Local Parameters for the path of the saved local recordings. - VLC media player is recommended for playing local recordings of 4K cameras. |
| / | Start/stop two-way audio. |
| / | Start/stop digital zoom. See Digital Zoom for details. |
| / | Start/stop capturing. See Capture for details. |
|  | Full screen. |
|  | Display/hide the PTZ Control, General Parameters tabs. |
|  | Distance for Optimum Multi-Sensor Splice(m) button, used to adjust the distance from the monitored object to the lens to get the optimal images. The value range is [2.0-100]m, the default is 10m. Adjust the splicing distance to optimize the splicing effect of the left and right screens. Examples are shown below: The distance is too small, causing double images.; - The distance is appropriate. The image is displayed correctly.; The distance is too large. The image looks smaller than normal.. NOTE! - This function is available on certain multi-sensor cameras. - You need to set the optimal splicing distance according to the actual scene. |

### Pixel Calculation

- Click  on the live view toolbar to enable pixel calculation.

- Adjust the position and size of the detection area. The pixel value changes with the size of the detection area.

To adjust the position of the detection area, point to a border of the area and drag it to the desired position.

To adjust the size of the detection area, point to a handle of the box and drag to resize it.

To redraw the detection area, click on the image and drag to draw a new one.

- To disable pixel calculation, click .

### Digital Zoom

- Click  on the live view toolbar to enable digital zoom.

- View the magnified area.

Click in the live view window and roll the wheel to zoom in or out on the image. Drag your mouse to view all the magnified area. To restore, right-click in the window.

Click in the live view window and drag your mouse to specify the area (rectangular area) to be magnified. Drag your mouse to view all the magnified area. To restore, right-click in the window.

- To exit, click .

## General Parameters

Set general parameters to view image effects directly.

On the live view page, click General Parameters.

# Playback

|  | NOTE! - Edge recordings refer to video recorded on storage media of cameras; local recordings refer to video recorded on a local PC. - Before you search for edge recordings, make sure that the camera has storage resources such as memory card, and the storage parameters in Storage are properly configured. - Recording playback and download functions are only available on certain models. - For dual-channel devices, you can set playback parameters for the channels separately. |
| --- | --- |

On the home page, click Playback.

## Playback Toolbar

| Button | Description |
| --- | --- |
|  | Adjust sound volume. Range: 1 to 100. |
|  | Start playback. |
|  | Pause playback. |
|  | Stop playback. |
|  | Clip video. |
|  | Save. |
|  | Adjust playback speed. The default speed is 1x. 2x plays faster, and 1/2x plays slower. -2x rewinds fast. |
|  | Take a snapshot. The snapshots are saved locally by default. You can change the storage location in Local Parameters. |
|  | Digital zoom. See Digital Zoom for details. |
| / | Zoom in/out on the time scale. You can also use the scroll wheel to zoom. |
| / | When the time scale is zoomed in, you can click or to view the previous or next section of the video. |
|  | Playhead. Drag the playhead to the desired point in the video. |
|  | Playback timeline, including two colors: Blue: Normal recording. Red: Alarm recording. To view alarm recordings, you need to configure alarm-triggered recording. See Alarm-triggered Actions for details. |

## Search and Playback

|  | NOTE! Certain cameras support simultaneous playback on two clients. |
| --- | --- |

- For a multi-channel camera, select the channel first.

- Select the date on the calendar.

- Click All Recording in the list and then choose the recording type you want to search.

- Click Search. Search results are displayed.

- Double-click an item in the list to play the recording. The playback stops till it reaches the end of the list or when you click  to pause.

## Recording Download

You can download videos or video clips in batches.

Download recordings

- Click Recording Download.

- Choose the recording type, set a time range, and then click Search. Search results are displayed.

- Click Browse… and set the download destination.

- Select the checkboxes for the recordings you want to download and click Download.

Download video clips

- Search for the video you want to clip. See Search and Playback.

- On the playback toolbar, click .

- Click on the timeline to specify the clip by setting the start and end time.

- Click . The clip turns cyan on the timeline.

- Click . The Recording Download page appears.

Click Browse… and set the download destination.

Select the checkbox for the clips you want to download and click Download.

# Photo

View the photo storage status. See Storage for photo storage policy.

|  | NOTE! This function is only available on cameras with storage capabilities. |
| --- | --- |

On the home page, click Photo.

| Item | Description |
| --- | --- |
| Refresh | Refresh the displayed content. |
| Export | Export the selected photos. |
| Delete | Delete the selected photos. |
| Export & Delete | Export the selected photos and delete them on the server. |
| Ascending Order | Arrange the items in chronological order. |
| Descending Order | Arrange the items in reverse chronological order. |
| Smart Server | Used to store smart snapshots. |
| Common Server | Used to store common snapshots. |

|  | NOTE! To allocate photo capacity, go to Setup > Storage > Storage. |
| --- | --- |

# Setup

## Local Parameters

Set local parameters for your PC, including smart, video, recording and snapshot.

|  | NOTE! The local parameters displayed may vary with camera model. |
| --- | --- |

- Go to Setup > Common > Local Parameters.

- Set local parameters as needed.

| Item | Description |
| --- | --- |

| Video | Display Mode | Set the display mode according to the network status, including Min. Delay, Balanced, and Fluent (from low delay to high delay). You may also customize the display mode as needed. |
| --- | --- | --- |
|  | Protocol | Set the protocol used to transmit media streams to be decoded by the PC, including TCP and UDP. |
| Recording and Snapshot | Recording | - Subsection By Time: Length of each local recording file. For example, 2 minutes. - Subsection By Size: Size of each local recording file. For example, 10MB. |
|  | Subsection Time (min)/Subsection Size (MB) | - Subsection Time (min): Available when Subsection By Time is selected. 1 to 60 minutes allowed. - Subsection Size (MB): Available when Subsection By Size is selected. 10 to 1024MB allowed. |
|  | When Storage Full | - Overwrite Recording: When the local recording capacity is full, older recordings are overwritten automatically. - Stop Recording: When the local recording capacity is full, recording stops automatically. |
|  | Total Capacity (GB) | - Allocate storage capacity for local recording. - Range: 1 to 1024GB. |
|  | Local Recording | - Set the file format for saving local recordings, including TS and MP4. |
|  | Files Folder | Set the location where snapshots and recordings are saved. Click Browse... to select the storage location. Click Open to quickly open the folder. NOTE! The maximum length of the directory is 260 bytes. If the limit is exceeded, recording or snapshot during live view will fail. |

- Click Save.

## Network

### Basic Config

Complete the basic network configuration so the camera can communicate with other devices on the network.

#### Ethernet

- Go to Setup > Network > Basic Config.

- Configure Ethernet parameters.

IPv4

Static address (configure IP manually)

Select Static from the Obtain IP Address drop-down list.

Enter the IP address, subnet mask, and default gateway address. Make sure that the IP address of the camera is unique in the network.

Click Save.

DHCP

DHCP (Dynamic Host Configuration Protocol) is enabled by default. If a DHCP server is deployed in the network, the camera can automatically obtain an IP address from the DHCP server.

  - Select DHCP from the Obtain IP Address drop-down list.

  - Complete the settings as shown below.

IPv6

Manual

  - Choose Manual.

  - Enter the IPv6 address, prefix length and default gateway. Make sure that the IPv6 address is unique in the network.

Set the MTU value, port type and operating mode.

MTU: Maximum transmission unit. Input an appropriate value. The greater the value, the higher the transmission efficiency, and the higher the delay.

Port Type: The default type is FE Port.

Operating Mode: The default is Auto-negotiation.

Click Save.

#### DNS

The DNS service is a globally distributed service that translates human readable domain names into numeric IP addresses. To use the DNS service, you need to configure DNS server information.

- Go to Setup > Network > Basic Config > DNS.

- Set the preferred and alternate DNS server addresses. The default settings are as follows.

#### DDNS

DDNS (Dynamic Domain Name Server) can map the dynamic IP address of the camera to a fixed domain name and so users can access the camera using the easy-to-remember domain name.

- Go to Setup > Network > Basic Config > DDNS.

- Enable DDNS Service.

- Select the DDNS type.

DynDNS/NO-IP: Third-party DDNS service provider. Enter the server address and domain name provided by your DDNS service provider.

MyDDNS: Enter a domain name and then click Test to check its availability.

- Click Save.

#### Port

- Go to Setup > Network > Basic Config > Port.

- You can use the default ports. If a port entered has been used for other services, you need to change the port to avoid conflict.

|  | CAUTION! If the HTTP port number you entered is being used by another service, a message indicating port conflict will appear. Ports 23, 81, 82, 85, 3260, and 49152 are reserved for other purposes and cannot be used. In addition to the above port numbers, the system can also dynamically detect other port numbers that are already in use. |
| --- | --- |

HTTP/HTTPS Port: If you change the HTTP/HTTPS port number, then you need to add the new port number after the IP address when logging in. For example, if the HTTP port number is set to 88, you need to use http://192.168.1.13:88 to log in to the camera.

RTSP Port: Real-Time Streaming Protocol port, enter an available port number.

- Click Save.

#### Port Mapping

Configure port mapping so users can access your camera on the LAN from the Internet.

- Go to Setup > Network > Port > Port Mapping.

- Enable Port Mapping.

- Select the mapping type.

UPnP

Auto: When enabled, the camera will negotiate external ports with the router automatically. Make sure UPnP is enabled on the router.

Manual: In this mode, you need to enter external ports manually and make sure the ports entered are available; otherwise, port mapping does not take effect.

Manual

If your router does not support UPnP, you need to set the external port numbers manually. Make sure the entered ports are available; otherwise, port mapping does not take effect.

Inactive will be displayed in the Status column if the port number entered is already in use by another service.

Click Save.

### Service Configuration

#### Email

Configure E-mail so that the camera can e-mail an alarm message to the specified email addresses when an alarm occurs.

- Go to Setup > Network > Service Config > E-mail.

- Set the sender and recipient information.

| Item | Description |
| --- | --- |
| Sender Name | Enter the device name. |
| Sender Address | Enter the device IP. |
| SMTP Server/SMTP Port | Enter the IP address and port number of the sender’s SMTP server. The default SMTP port number is 25. |
| TLS/SSL | When enabled, emails will be encrypted by TLS or SSL during transmission. NOTE! If SMTP supports TLS/SSL, it tries SSL first to establish a secure connection for email sending. |
| Snapshot Interval | Choose a snapshot interval: 2s, 3s, 4s, or 5s. |
| Attach Image | When enabled, an alarm e-mail with attached snapshots (3 by default) taken at set interval will be sent after an alarm occurs. For some alarm types, only one snapshot will be taken and there’s no need to set snapshot interval. - Select the Attach Image checkbox. Enable Snapshot and set the snapshot resolution as needed. There’s no need to set snapshot schedule. |  |
| Server Authentication | Enable SMTP server authentication to secure e-mail transmission. |  |
| Username/Password | Enter the username and password of the SMTP server. NOTE! The email only shows the sender name. Username will not be displayed. The password allows special characters. |  |
| Recipient Name/Address | - Enter the recipient names and email addresses. Click Test to test the email sending function. |  |

- Click Save.

#### SNMP

The Simple Network Management Protocol (SNMP) allows the camera to be remotely managed by a management server. The camera can be configured to support SNMP and send messages to the management server for important events or status change.

- Go to Setup > Network > Service Config > SNMP.

- Click On to enable SNMP.

|  | NOTE! On certain device models, SNMP is enabled after an upgrade to the latest version. It is normal. |
| --- | --- |

- Set SNMP parameters.

SNMPv3

|  | NOTE! Before you enable SNMPv3, make sure SNMPv3 is supported both on your camera and the server. |
| --- | --- |

| Item | Description |
| --- | --- |
| SNMP Type | The default is SNMPv3. |
| Password | Set an authentication password. |
| Confirm | Confirm the authentication password by entering it again. |
| Password | Set an encryption password. |
| Confirm | Confirm the encryption password by entering it again. |
| Trap Server Address | It is filled in automatically after you complete management server configuration. |
| SNMP Port | The default is 161. You may change it as needed. |

SNMPv2

| Item | Description |
| --- | --- |
| SNMP Type | A message will appear to remind you of potential risks and prompt for your confirmation after you choose SNMPv2. To choose SNMPv2 anyway, click OK to ignore the reminder. |
| Read Community | The default is public. You may change it as needed, but make sure you also change it on the server; otherwise, authentication will fail. |
| Trap Server Address | Go to Setup > Network > Platform Access > Management Server to configure the server address. |
| SNMP Port | The default is 161. You may change it as needed. |

- Click Save.

#### QoS

QoS (Quality of Service) can alleviate network delay and network congestion by prioritizing different services.

- Go to Setup > Network > Service Configuration > QoS.

- Assign a priority level (0 to 63) for each service. The greater the value, the higher the priority. For example, the camera first ensures smooth audio and video in the case of network congestion.

|  | NOTE! The same QoS rules must also be configured on the router or network switch. |
| --- | --- |

- Click Save.

## Video & Audio

For dual-channel devices, you can set video and audio parameters for the channels separately.

### Video

#### Video

- Go to Setup > Video & Audio > Video.

- Select a capture mode for your camera.

The Extended Encoding function is available only when the capture mode is greater than 8MP.

After you change the capture mode, the encoding settings will be reset to defaults and some models of cameras will restart.

- Set stream parameters.

The streams are independent of each other and can be set with different resolutions, frame rates, video compression formats, etc. Only the main stream supports full resolution.

| Item | Description |
| --- | --- |
| Video Compression | Select a video compression standard for your camera: H.265, H.264 or MJPEG. NOTE! When H.265 or H.264 is selected, Image Quality is not available; When MJPEG is selected, Bit Rate, I Frame Interval, Smoothing, SVC and Smart Encoding are not available. The bit rate restores to the default when you switch between H.264 and H.265. |
| Resolution | Select a video resolution for your camera. The higher the resolution, the clearer the image. |
| Frame Rate(fps) | Select the frame rate. NOTE! To ensure image quality, the frame rate shall not be greater than the reciprocal of the shutter speed. |
| Bit Rate(Kbps) | Set the bit rate. Range: 128 to 16384. NOTE! The bit rate range may vary with device model. |
| Bitrate Type | Select the bitrate type. - CBR: The camera keeps a specific bit rate by varying the quality of video streams. - VBR: The camera keeps the quality of video streams as constant as possible by varying the bit rate. |
| Image Quality | Configurable when Bitrate Type is set to VBR. The closer the slider is to Quality, the higher the bit rate, and the higher the image quality. The closer the slider is to Bit Rate, the lower the bit rate, and the image quality will be affected. |
| I Frame Interval | Set the number of frames between I-frames. A shorter interval presents better image quality but consumes more bandwidth and storage. |
| GOP | Group of Pictures, defines the basic pattern of the video stream encoded with I and P frames. |
| Smoothing | Set the smoothness of the video stream. Drag the slider to choose whether smoothness or clarity takes precedence. NOTE! Smoothing is recommended for fluent video in a poor network environment. |
| SVC | SVC (Scalable Video Coding) enables a video stream to be broken into multiple layers of resolution, quality and frame rate, reducing bandwidth consumption without compromising the image quality. |
| Smart Encoding | Select a mode. Basic Mode: Reduces the bit rate by about 25%. Advanced Mode: Reduces the bit rate by about 50%. |

- Set the BNC output format, PAL or NTSC.

- Click Save.

### Snapshot

Configure basic snapshot parameters and scheduled snapshot.

- Go to Setup > Video & Audio > Snapshot.

|  | NOTE! For dual-channel devices, you can set snapshot parameters for the channels separately. When you configure e-mail and FTP, you only need to enable Snapshot and set the resolution and maximum size; it’s unnecessary to configure the scheduled snapshot. |
| --- | --- |

- Enable Snapshot, set the resolution and maximum size of snapshots.

- Set the snapshot mode.

Schedule: Set a snapshot time. For example, with Snapshot Interval set to 20s, Number to Snapshot set to 3, and snapshot time set to 16:00:00, the camera will take a snapshot at 16:00:00, 16:00:20 and 16:00:40. You can click  to add a snapshot time by choosing from the list or by adjusting based on a listed time.

- To delete a snapshot time, click .

Repeat: Set a snapshot interval. For example, with snapshot plan set to 16:00:00 to 20:00:00 on Monday, Repeat Interval set to 120s, Snapshot Interval set to 20s, and Number to Snapshot set to 2, the camera will take a snapshot at 16:00:00, 16:00:20, 16:02:00 and 16:02:20.

    - Select Repeat and set the repeat interval. A valid repeat interval is an integer within the range of 1 to 86400. The snapshot interval * number of snapshots to take cannot be greater than the interval (Snapshot Interval * Number of Snapshot cannot be greater than Interval).

Select the Enable Snapshot Plan checkbox and set the snapshot plan. See Arming Schedule for details. A 24/7 snapshot plan is enabled by default.

|  | NOTE! The time periods cannot overlap. Up to 4 time periods are allowed. |
| --- | --- |

Set the snapshot interval (unit: second) and number to snapshot (1, 2, or 3). For example, if the interval is set to 1s and the number to snapshot is set to 2, the camera will take 2 snapshots (take one first and then take another after 1 second).

Click Save.

### Audio

#### Audio

- Go to Setup > Video & Audio > Audio.

- Set audio input parameters.

| Item | Description |
| --- | --- |
| Audio Input | Enable/disable audio input. NOTE! If audio is not needed, it is recommended to turn it off to improve camera performance. |
| Access Mode | Choose the audio input mode; Line/Mic or RS485. NOTE! This function is not available on dual-channel cameras. |
| Input Volume | Drag the slider or input a value to set the input volume. |
| Audio Compression | Choose the audio compression format: G.711U or G.711A. |
| Sampling Rate(KHz) | Choose a sampling rate, 8KHz or 16KHz. The higher the sampling rate, the better the sound quality. |
| Noise Suppression | This function is enabled by default. It can reduce noises and improve audio output quality. |
| Channel 1/Channel 2 | Select the Enable checkbox to enable audio input for Channel 1 or Channel 2 (if available). Only one channel can be enabled. Channel 1 supports Mic (default) and Line. Channel 2 is available on certain models only. |

- Set audio output parameters

| Item | Description |
| --- | --- |
| Audio Output | Choose an audio output mode, Line. Line: A speaker or an earphone needs to be connected. |
| Output Volume | Set the output volume using the slider. |

- Click Save.

### ROI

Region of Interest (ROI) ensures image quality of specified areas on the image at low bit rate.

- Go to Setup > Video & Audio > ROI.

- Set ROI areas.

  - Click  to add a ROI area. The area is a rectangle by default. Up to 8 areas are allowed.

Adjust the position and size of the area or draw an area as needed.

Adjust the position and size of the area.

Point to a border of the area and drag it to the desired position.

Point to a handle of the area and drag to resize it.

Draw an area.

- Click on the image and drag to draw an area.

|  | NOTE! When setting the ROI, you can click to lock the scene to prevent pan/tilt movement caused by triggered detection rules, and click to unlock the scene after finishing setting the ROI. |
| --- | --- |

### Media Stream

#### Media Stream

You can configure a media stream for your camera so that media contents from the camera such as audio and video can be transmitted over the network and played immediately on a third-party client rather than being downloaded first.

- Go to Setup > Video & Audio > Media Stream.

- Click  to add a media stream.

- Complete the media stream settings.

| Item | Description |
| --- | --- |
| Stream Profile | Select a stream type for the camera to transmit media contents to a third-party client. |
| Destination IP | Enter the IP address of the device receiving media streams. |
| Protocol | The camera transmits data to a third-party client through the specific protocol. The default protocol is RTMP. |
| Persistent | Set whether to automatically establish the configured media stream after the camera restarts. |

- Click OK.

#### RTSP Multicast

RTSP multicast allows third-party players to request RTSP multicast media streams from the camera through the RTSP protocol.

- Go to Setup > Video & Audio > Media Stream > RTSP Multicast Address.

- Set the multicast address and port number (multicast address range: 224.0.1.0 to 239.255.255.255, port number range: 0 to 65535).

- Click Save.

## Image

### Image

#### Scenes

A scene is a collection of image parameters preset in the camera for users to choose according to the actual scene.

- Go to Setup > Image > Image.

- Click Scenes.

- Set the parameters.

| Item | Description |
| --- | --- |
| Current | Shows the scene that is currently in use. NOTE! To apply a scene, click the corresponding radio button in the Current column. The scene is selected as the current scene, and the corresponding image parameters apply automatically. |
| Image Scene Template | The camera provides multiple scenes for you to choose according to the actual scene. After you select a scene, the corresponding image settings apply automatically (you may also fine-tune image settings as needed). - Common: For outdoor scenes. - Indoor: For indoor scenes. - Road Highlight Compensation/Park Highlight Compensation: For capturing license plates in roads, parks, etc. - WDR: For high-contrast situations. - Custom: For customizing a scene. - Test: For testing. - Standard: For common scenes, indoor and outdoor. - Vivid: Provides extra saturation based on the Standard scene. - Bright: Provides extra brightness based on the Standard scene. - Starlight: For lowlight environments. - Face: For capturing faces in motion in complicated scenes. - Person And Vehicle: For monitoring motor vehicles, non-motor vehicles, and pedestrians in road scenes. - Intrusion Prevention: For perimeter protection scenes. |
| Image Scene Name | Choose scene names corresponding to the scene templates. The scene names will be used in image scene switching (see Image Scene Switch). |

#### Image Enhancement

- On the Image page, click Image Enhancement.

- Set the image enhancement parameters.

| Item | Description |
| --- | --- |
| Brightness | The overall lightness or darkness of the image. Low brightness; High brightness. |
| Saturation | The intensity or vividness of colors in the image. Low saturation; High saturation. |
| Contrast | The difference between the lightest and darkest tones in the image. Low contrast; High contrast. |
| Sharpness | The definition of edges in the image. Low sharpness; High sharpness. |
| 2D Noise Reduction | Reduce noise by individually analyzing each frame, which may cause image blur. |
| 3D Noise Reduction | Reduce noise by analyzing the difference between successive frames, which may cause image smearing or ghosting. |
| Image Rotation | The rotation of the image. Normal; Flip vertical. Flip horizontal; 180°. 90° clockwise; 90° anti-clockwise. |

To restore defaults, click Default.

#### Exposure

|  | NOTE! The exposure settings may vary with device model. The default settings are scene-adaptive. Use default settings unless modification is necessary. |
| --- | --- |

- On the Image page, click Exposure.

- Set the exposure parameters.

| Item | Description |
| --- | --- |
| Exposure Mode | Select the exposure mode. - Automatic: The camera automatically set the optimum shutter speed according to the scene. - Custom: User can set exposure parameters as needed. - Shutter Priority: The camera adjusts shutter as priority to adjust the image quality. - Iris Priority: The camera adjusts iris as priority to adjust the image quality. - Indoor 50Hz: Reduce stripes by adjusting the exposure time. NOTE! - Stripe effect: The high-contrast condition in an image caused by uneven light energy received by the sensor. - Using this mode in brighter environments aids in adjusting the stripe effect in the image with linear stripe suppression. - Indoor 60Hz: Reduce stripes by adjusting the exposure time. NOTE! Using this mode in brighter environments aids in adjusting the stripe effect in the image with linear stripe suppression. - Manual: Fine-tune image quality by setting shutter, gain and iris manually. - Low Motion Blur: Control the minimum shutter to reduce motion blur in faces captured in motion. |
| Shutter(s) | Shutter is used to control the light that comes into the lens. A fast shutter speed is ideal for scenes in quick motion. A slow shutter speed is ideal for scenes that change slowly. NOTE! This parameter is configurable when Exposure Mode is set to Manual, Shutter Priority, or Custom. - If Slow Shutter is disabled, the reciprocal of the shutter speed must be greater than the frame rate. |
| Gain | Control image signals so that the camera can output standard video signals in different light conditions. NOTE! This parameter is configurable when Exposure Mode is set to Manual or Custom. |
| Slow Shutter | Increase image brightness in low light conditions. NOTE! This parameter is configurable when Exposure Mode is not set to Iris Priority and Image Stabilization is disabled. |
| Slowest Shutter | Set the slowest shutter speed for exposure. |
| Compensation | Adjust the compensation value as required to achieve the desired image effect. NOTE! This parameter is configurable when Exposure Mode is not set to Manual. |
| Linear Stripe Suppression | Adjust the linear stripes in the image. Range: 1 to 9, the default is 5. The greater the value, the more obvious the linear stripe suppression effect is, but it may cause overexposure in the image. Please configure this according to the actual scene. NOTE! This parameter is configurable when Exposure Mode is set to Indoor 50Hz or Indoor 60Hz. |
| Restore Auto Exposure(min) | Set the duration for the camera to restore automatic exposure mode. |
| Metering Control | Perform luminance statistics on the images captured by the device, automatically adjust the exposure value, and output properly exposed images with optimal brightness. The default is the Center-Weighted Average Metering, you may configure this according to the actual scene. - Center-Weighted Average Metering: Measure light mainly in the central part of the image. - Evaluative Metering: Measure light in the specified area of the image, suitable for scenes where the target and the background contrast widely. - Spot Metering: Measure light in a small spot, smaller than evaluative metering, with more accurate light control. If used in brighter areas, the metered area will be properly exposed but darken other areas. If used in darker areas, the metered area will be properly exposed (unable to increase the brightness of the spot) and brighten other areas. - Face Metering: Adjust image quality in poor lighting or back lighting conditions by controlling the brightness of captured faces in face scenes. NOTE! This parameter is configurable when Exposure Mode is not set to Manual. |
| Day/Night Mode | - Automatic: The camera automatically switches between day mode and night mode according to the ambient lighting condition to output optimum images. - Day: The camera outputs high-quality images in daylight conditions. - Night: The camera outputs high-quality images in low-light conditions. - Input Boolean: The camera switches between day mode and night mode according to the Boolean value input from a connected third-party device. NOTE! The Input Boolean option is only available on certain models. |
| Day/Night Sensitivity | Light threshold for switching between day mode and night mode. A higher sensitivity value means that the camera is more sensitive to the change of light and is therefore more easily to switch between day mode and night mode. NOTE! This parameter is configurable when Day/Night Mode is set to Automatic. |
| Day/Night Switching(s) | Set the length of time before the camera switches between day mode and night mode after the switching conditions are met. NOTE! This parameter is configurable when Day/Night Mode is set to Automatic. |
| WDR | - Enable: Ensure clear images in high contrast conditions. You may need to manually adjust the WDR level. - Smart/Auto: In typical WDR scenes, the camera enables WDR automatically; in other scenes, the camera displays the original scene. NOTE! Typical WDR scenes: high contrast situations (poorly lit halls), back lighting situation (toll booths), highlight scenes, etc. This parameter is configurable when Exposure Mode is set to Automatic, Custom, Shutter Priority, Indoor 50Hz or Indoor 60Hz and when Image Stabilization and Defog are disabled. |
| WDR Level | Adjust the WDR level. NOTE! Level 7 or higher is recommended if there is a high contrast between the bright and dark areas in the scene. In the case of low contrast, it is recommended to disable WDR or use level 1 to 6. |
| WDR On/Off Sensitivity | When WDR is set to Automatic, adjust the parameter to change the WDR switching sensitivity. |
| Suppress WDR Stripes | Suppress the stripes in the image caused by the flickering light in WDR mode. When enabled, the camera automatically adjusts the shutter and frequency to minimize stripes. |

To restore defaults, click Default.

#### Smart Illumination

- On the Image page, click Smart Illumination.

- Enable Smart Illumination.

- Set the smart illumination parameters.

| Item | Description |
| --- | --- |
| Illumination Mode | - Infrared: The camera uses infrared light illumination. - White Light /Smart White Light: The camera uses white light illumination. - Warm Light: The camera uses warm light illumination. - Laser: The camera uses laser light illumination. NOTE! Before you select Warm Light, please set the Port Mode to Illumination (go to Setup > System > Ports & Devices > Serial Port). This parameter is configurable when Control Mode is set to Custom Level. Range: 0 to 100. 0 is the weakest, 100 is the strongest. When Smart Illumination is enabled and illumination mode is set to Smart White Light: - Alarm-trigger enabled: when an alarm is triggered, during the daytime, the white light blinks; at night, the white light is on steadily, otherwise, the white light is on faintly. - Alarm-trigger N/A or disabled: when an alarm is triggered, during the daytime, the white light is off; at night, the white light is on steadily, otherwise, the white light is on faintly. |
| Control Mode | - Global Mode: The camera automatically adjusts illumination and exposure to achieve the balanced image effect. Some areas might be overexposed if you select this option. This option is recommended if you focus on the monitoring range and image brightness. - Overexposure Restrain: The camera automatically adjusts illumination and exposure to avoid regional overexposure. Some areas might be dark if you select this option. This option is recommended if you focus on the clarity of the monitoring center area. - Road: This mode offers a strong overall illumination and is recommended for monitoring wide-range scenes, for example, road. - Park: This mode offers a uniform illumination and is recommended for monitoring small-range scenes with many obstacles, for example, park. Custom Level: This mode allows you to manually control the intensity of illumination. - Custom Level(Always On): In this mode, the illumination is always on. |
| Illumination Level | Set the intensity of the illuminator. The greater the value, the higher the intensity. 0 is off. - Near-illumination Level: Recommended for near focus scenes. - Mid-illumination Level: Recommended for medium distance focus scenes. - Far-illumination Level: Recommended for far focus scenes. NOTE! This parameter is configurable when Control Mode is set to Custom Level. |

To restore defaults, click Default.

#### White Balance

White balance is used to eliminate unnatural color casts in images under different color temperatures for optimal color reproduction.

- On the Image page, click White Balance.

- Set the white balance parameters.

| Item | Description |
| --- | --- |
| White Balance | Adjust the red and blue gains of the image to remove unrealistic color casts. - Auto/Auto 2: Automatically adjust the red and blue gains according to the lighting conditions. If there are still color casts in Auto mode, try Auto 2 mode. - Fine Tune: Manually adjust the red and blue offsets. Sodium Lamp: Automatically adjust the red and blue gains for optimal color reproduction in sodium light sources. - Outdoor: Recommended for outdoor scenes where the color temperature varies widely. - Locked: Keep the current color temperature. |
| Red/Blue Offset | Set the red/blue offset. NOTE! This parameter is configurable when White Balance is set to Fine Tune. |

To restore defaults, click Default.

#### Defog

Defog is used to improve image visibility in foggy, hazy and other low-visibility scenes.

- On the Image page, click Advanced.

|  | NOTE! This function is only available when WDR is disabled. |
| --- | --- |

- Set the defog parameters.

| Item | Description |
| --- | --- |
| Defog | Select the defog mode, including Automatic, On, and Off. In Automatic mode, the camera automatically adjusts the defog intensity according to the fog concentration for clear images. |
| Defog Intensity | Adjust the defog intensity. In a heavy-fog environment, the higher the defog level, the clearer the image; in a fog-free or light-fog environment, there is not much difference between levels 1 to 9. NOTE! Optical defog is available on certain models. To enable optical defog, select On and set the defog intensity to 6 or higher, or select Automatic. Optical defog is automatically turned on in thick fog, and the image changes from color to black and white. |

To restore defaults, click Default.

#### Engineering Parameters

During lens rotation, the camera may lose focus, causing unclear images. In this case, you can initialize the lens to refocus.

- Go to Setup > Image > Engineering.

- Reset lens.

To reset manually, click Reset.

To reset automatically, select the Enable Auto Initialization checkbox and set an execution time.

- Click Save.

### OSD

On Screen Display (OSD) are characters displayed with video images, for example, camera name, date and time.

|  | NOTE! This function may vary with device model. For dual-channel cameras, you can set OSD parameters for the channels separately. |
| --- | --- |

Configure OSD displayed on the live video.

- Go to Setup > Image > OSD .

- Configure parameters in the OSD Toolbar

|  | NOTE! OSD operations supported may vary with device model. |
| --- | --- |

| Item | Description |
| --- | --- |
| Enable | Select the checkboxes in the Enable column to overlay the corresponding OSD contents on live video. |
| Overlay OSD Content | Set OSD content you want to overlay. Point to the OSD content, click , select the OSD content from the drop-down list or customize it. NOTE! An OSD content takes effect only when the Enable checkbox is selected. Some models allow multiple OSD contents in one overlay area. |
| X-Axis/Y-Axis | Specify the exact position of the OSD by entering the X and Y coordinates. The top left corner of the image is the origin (0, 0), the horizontal axis is the X-axis, and the vertical axis is the Y-axis. NOTE! You can also drag an OSD to the desired position: point to the OSD box in the preview window, and drag when the cursor shape changes. |

- Overlay OSD Contents

Custom: Customizes the OSD contents.

Date & Time: Displays the current date and time of the camera according to the set format (content style), for example, Friday, 25 March, 2022 14:25.

- PTZ Coordinates: Specifiy the exact position of the PTZ camera. For example, P: 165.42°, T: 1.49°. P stands for horizontal coordinates, T stands for vertical coordinates.

Time: Displays the current time of the device.

Date: Displays the current date of the device.

Network Port: The IPC will receive and parse the network port information in correct format and display the information in the live view window.

|  | NOTE! To disable an OSD, clear the OSD content or clear the corresponding Enable checkbox. Some models allow multiple OSDs. |
| --- | --- |

- Set the OSD display style.

| Item | Description |
| --- | --- |
| Effect | Choose a display effect: Background, Stroke, Hollow, or Normal. |
| Font Size | Choose a font size: X-large, Large, Medium, or Small. |
| Font Color | - Click to choose a font color. |
| OSD Inverse | - When enabled, the OSD content color is oppsite to the live view color. |
| Min. Margin | - Choose a minimum distance between the OSD area and the edge of image: None: Zero margin. Single: One-character margin. Double: Two-character margin. |
| Date Format | Choose a date format: dd/MM/yyyy, MM/dd/yyyy, etc. |
| Time Format | Choose a time format: HH:mm:ss, HH:mm:ss.aaa, hh:mm:ss tt, or hh:mm:ss.aaa tt. |

### Privacy Mask

Privacy mask is used to cover certain areas on the image for privacy, for example, ATM keyboard.

|  | NOTE! This function may vary with device model. For dual-channel devices, you can set privacy mask parameters for the channels separately. |
| --- | --- |

- Go to Setup > Image > Privacy Mask.

- Add a privacy mask.

  - Click Add. The privacy mask is a rectangle by default.

Adjust the position and size of the mask or draw a mask as needed.

Adjust the position and size of the mask.

Point to a border of the mask and drag it to the desired position.

Point to a handle of the mask and drag to resize it.

Draw a mask.

Rectangle: Click on the image and drag to draw a rectangle.

## Smart

On the Intelligent > Smart page, you can select the smart event to be monitored and click  to configure relevant parameters.

The smart events supported by the device and the parameters supported by the events may vary with device model.

### Alarm-triggered Actions

Configure linkage actions to be triggered when an alarm occurs.

Perimeter protection, exception detection, object detection, people flow counting, and auto tracking support alarm-triggered actions (also known as linkage actions).

#### Conventional alarm-triggered actions

| Item | Description |
| --- | --- |
| Upload to FTP | The camera uploads snapshots to the specified FTP server when an alarm occurs. - Configure FTP and Snapshot. - Select the Upload to FTP checkbox to enable this function. |
| Send E-mail | The camera sends snapshots to the specified email addresses when an alarm occurs. - Configure E-mail and Snapshot. - Select the Send Email checkbox to enable this function. |

#### Alarm output

| Item | Description |
| --- | --- |
| Alarm Output | Connect the external alarm output devices, such as alarm lights and alarm bells, etc., to the camera's ALARM OUT interfaces. The camera triggers alarm output devices when an event occurs. A->1: A refers to the ALARM OUT interface on the device, 1 means the first ALARM OUT interface. - Configure Alarm Output. - Select the Alarm Output checkbox to enable this function. |

#### Alarm-triggered storage

| Item | Description |
| --- | --- |
| Recording Edge Storage | The camera saves alarm recordings to its memory card or NAS when an alarm occurs. - Configure Memory Card or Network Disk. - Select the Recording Edge Storage checkbox. |
| Image Edge Storage | The camera saves alarm snapshots to its memory card or NAS when an alarm occurs. - Configure Memory Card or Network Disk. - Select the Image Edge Storage checkbox. |

### Arming Schedule

You can set an arming schedule to determine when the camera performs detection.

Draw a schedule

To set an armed period, click Armed, and then click or drag on the schedule to select the time cells you want to enable arming. To set a disarmed period, click Unarmed, and then click or drag on the schedule to select the time cells you want to disable arming.

Edit a schedule

Click Edit, set the arming time, and then click OK.

|  | NOTE! Up to 4 time periods are allowed per day. The time periods cannot overlap. To apply the same time settings to other days, select the desired day(s), and then click Copy. |
| --- | --- |

### Cross Line Detection

Cross line detection detects objects crossing a user-specified virtual line in a specified direction. The camera reports an alarm when the detection rule is triggered.

- Go to Setup > Intelligent > Smart.

- Select Cross Line and click  to configure it.

- Add a detection rule.

  - Click  to add a detection line. Up to 4 detection rules are allowed.

  - Adjust the position and length of the line or draw a line as needed.

Adjust the position and length of the line.

Point to the line and drag it to the desired position.

Point to a handle of the line and drag to resize it.

Draw a line.

- Click on the image and drag to draw a line.

|  | NOTE! When editing detection rules, you can click to lock the scene to prevent pan/tilt movement caused by triggered detection rules, and click to unlock the scene after you finish editing detection rules. |
| --- | --- |

- Set the detection rule.

| Item | Description |
| --- | --- |
| Trigger Direction | - Select the direction from which the object crosses the line to trigger an alarm. A->B: The camera reports a cross line alarm when it detects an object crossing the line from A to B. B->A: The camera reports a cross line alarm when it detects an object crossing the line from B to A. A<->B (default): The camera reports a cross line alarm when it detects an object crossing the line from A to B or from B to A. |
| Sensitivity | Set the detection sensitivity. The higher the sensitivity, the more likely cross line behaviors will be detected, and the more likely false alarms will occur. |

- Set the alarm-triggered actions and arming schedule. See Alarm-triggered Actions and Arming Schedule for details.

- Click Save.

### Intrusion Detection

Intrusion detection detects objects entering a user-specified area and staying for a preset time. The camera reports an alarm when the detection rule is triggered.

- Go to Setup > Intelligent > Smart.

- Select Intrusion and click  to configure it.

- Add a detection rule.

  - Click  to add a detection area. The detection area is a hexagon by default. Up to 4 detection rules are allowed.

  - Adjust the position and size of the area or draw an area as needed.

Adjust the position and size of the area.

Point to a border of the area and drag it to the desired position.

Point to a handle of the area and drag to resize it.

Draw an area.

- Click on the image and drag to draw a line. Repeat the action to draw more lines to form an enclosed shape as needed. Up to 6 lines are allowed.

|  | NOTE! When editing detection rules, you can click to lock the scene to prevent pan/tilt movement caused by triggered detection rules, and click to unlock the scene after you finish editing detection rules. |
| --- | --- |

- Set the detection rule.

| Item | Description |
| --- | --- |
| Time Threshold(s) | - Set how long the object stays in the detection area to trigger an intrusion alarm. If an object stays in the detection area for the set time, an intrusion alarm will be triggered. |
| Sensitivity | Set the detection sensitivity. The higher the sensitivity, the more likely intrusion behaviors will be detected, and the more likely false alarms will occur. |

- Set the alarm-triggered actions and arming schedule. See Alarm-triggered Actions and Arming Schedule for details.

- Click Save.

## Alarm

Configure the alarm function, so the camera can report alarms when an event occurs. Configure alarm linkage, so the camera can trigger other devices to perform specified actions when an event occurs.

|  | NOTE! The supported alarms and linkage actions (or trigger actions) may vary with camera model. |
| --- | --- |

### Common Alarm

#### Motion Detection

|  | NOTE! The icon appears in the upper right corner of the image when a motion detection alarm occurs. The function may vary with device model. |
| --- | --- |

- Go to Setup > Events > Common Alarm.

- Choose the detection mode, including basic mode and ultra mode.

Basic Mode

The camera detects motions in specified detection areas or grids on the image and will take snapshots and report an alarm when detection rules are triggered.

Detection area

  - To add a detection area, click , then a rectangle appears on the image. Up to four detection areas are allowed.

  - Adjust the position, size and shape of the rectangle detection area, or draw a new one.

Point to a border of the area and drag it to the desired position.

Point to a handle of the area and drag to resize it.

Click anywhere on the image, and then drag to draw a new area.

  - Set detection rules.

| - Item | - Description |
| --- | --- |
| - Sensitivity | - Drag the slider to adjust detection sensitivity. - The higher the sensitivity level, the higher the detection rate of small motions, and the higher the false alarm rate. Set based on the scene and your actual needs. |
| - Object size | - Drag the slider to set object size. - Object size: The ratio of the size of the detected object to the size of the detection area. An alarm is triggered when the ratio reaches the set value. To detect motion of small objects, you need to draw a small detection area separately. - Motion detection results of the current detection area are shown below in real time. The red means motions that have triggered a motion detection alarm. The height of the lines indicates the extent of motion. The density of the lines indicates the frequency of motion. The higher a line, the greater the extent. The denser the lines, the higher the frequency. |

  - Set Suppress Alarm to avoid receiving the same alarms within a certain length of time (alarm suppression time). For example, alarm suppression time is set to 5s, after an alarm is reported:

If no motion is detected within the next 5s, new alarms can be reported after 5s when the alarm suppression time (5s) is over.

If motion is detected within the next 5s, the alarm suppression time recounts from the time of the last alarm, and new alarms can be reported when the alarm suppressions time (5s) is over.

Grid detection

  - Set grid detection areas (covered by grid), which is by default the whole screen.

  - Edit detection areas as needed.

Click or drag on grid areas to erase grids.

Click or drag on blank areas to draw grids.

  - Drag the slider to adjust detection sensitivity.

- The higher the sensitivity level, the higher the detection rate of small motions, and the higher the false alarm rate. Set based on the scene and your actual needs.

  - Set Suppress Alarm to avoid receiving the same alarms within a certain length of time (alarm suppression time). For example, alarm suppression time is set to 5s, after an alarm is reported:

If no motion is detected within the next 5s, new alarms can be reported after 5s when the alarm suppression time (5s) is over.

If motion is detected within the next 5s, the alarm suppression time recounts from the time of the last alarm, and new alarms can be reported when the alarm suppression time (5s) is over.

- Set alarm linkage and an arming schedule. See Alarm-triggered Actions and Arming Schedule for details.

- Click Save.

Ultra Mode

The camera detects motions of motor vehicles, non-motor vehicles and pedestrians in specified detection areas and will take snapshots and report an alarm when detection rules are triggered.

- Set detection rules.

  - To add a detection area, click , then a rectangle appears on the image. Up to four detection areas are allowed.

  - Adjust the position, size and shape of the rectangle detection area, or draw a new one.

Point to a border of the area and drag it to the desired position.

Point to a handle of the area and drag to resize it.

Click anywhere on the image, and then drag to draw a new area.

  - Drag the slider to adjust detection sensitivity.

- The higher the sensitivity level, the higher the detection rate of small motions, and the higher the false alarm rate. Set based on the scene and your actual needs.

  - Select the detection object.

- Set alarm linkage and an arming schedule. See Alarm-triggered Actions and Arming Schedule for details.

- Click Save.

#### Tampering Detection

The camera triggers a tampering alarm after the lens is blocked for a certain length of time.

- Go to Setup > Events > Common Alarm > Tampering Detection.

- Select Enable Tampering Detection.

- Set detection rules.

  - Drag the slider to adjust detection sensitivity. The higher the sensitivity level, the higher the detection rate, and the higher the false alarm rate. Set based on the scene and your actual needs.

  - Set the duration of lens blocking. The camera reports an alarm when the duration of lens blocking exceeds the set value. Set based on the scene and your actual needs.

- Set alarm linkage and an arming schedule. See Alarm-triggered Actions and Arming Schedule for details.

- Click Save.

#### Audio Detection

The camera monitors input audio signals and triggers an audio detection alarm when an exception is detected. Make sure an audio collection device (e.g. sound pickup) is connected, and audio detection is enabled (see Audio).

When audio input mode is Line/Mic.

- Go to Setup > Events > Common Alarm > Audio Detection.

- Enable Audio Detection.

- Set audio detection rules.

| Item | Description |
| --- | --- |
| Detection Type | - Sudden Rise: Detects sudden rising sound volume, and triggers an alarm when the rise of volume exceeds the difference. - Sudden Fall: Detects sudden falling sound volume, and triggers an alarm when the fall of volume exceeds the difference. - Sudden Change: Detects sudden rising and falling sound volume, and triggers an alarm when the rise or fall of volume exceeds the difference. - Threshold: Triggers an alarm when the volume exceeds the threshold. |
| Difference/Threshold | - Difference: The difference between two sound volumes. The camera triggers an alarm when the rise or fall of volume exceeds the difference (range: 0-400). This parameter is applicable when the detection type is Sudden Rise, Sudden Fall, or Sudden Change. - Threshold: The camera triggers an alarm when the sound volume exceeds the threshold (range: 0-400). This parameter is applicable when the detection type is Threshold. |
| Diagram of relative audio intensity | - Audio detection results are displayed and updated in real time. You can control the display progress by clicking the Start/Stop button. - The scales are used to measure sound volume. Gray indicates relative sound intensity. Red means sound volume that has triggered alarms. |

Set alarm linkage and an arming schedule. See Alarm-triggered Actions and Arming Schedule for details.

- Click Save.

#### Alarm Input

The camera can receive alarms from external third-party devices such as infrared detectors, smoke detectors, etc. After alarm input is configured, the third-party device can send signals to the camera after an event occurs.

- Go to Setup > Events > Common Alarm > Alarm Input.

- Choose an alarm input from the drop-down list.

- The number of alarm inputs available may vary with camera model. For example, if the camera has two alarm inputs on the tail cable, you can configure alarm input 1 and alarm input 2 separately.

- Configure alarm input.

| Item | Description |
| --- | --- |
| Alarm Name | - The default name is the alarm input channel ID. You rename it as needed. |
| Alarm ID | - Set an alarm ID as you need. |
| Alarm Type | Set the alarm type according to the alarm input device. - If the alarm input device is normally open (N.O.), choose N.C.. - If the alarm input device is normally closed (N.C.), choose N.O.. |
| Alarm Input | - Click On to enable Alarm Input. |

- Set alarm linkage and an arming schedule. See Alarm-triggered Actions and Arming Schedule for details.

- Click Save.

#### Alarm Output

The camera can output alarms to external third-party devices such as alarm bell, buzzer, etc. After alarm output is configured, the camera can output alarm signals when an alarm (such as motion detection alarm, tamping alarm) occurred and trigger the third-party device to perform certain actions.

- Go to Setup > Events > Common Alarm > Alarm Output.

- Choose an alarm output from the drop-down list. The number of alarm outputs available may vary with camera model.

- Configure alarm output parameters.

| Item | Description |
| --- | --- |
| Alarm Name | - The default name is the alarm output channel ID. You can rename it as needed. |
| Default Status | Choose the default status. The default is N.O.. - If the external alarm device is normally open (N.O.), choose N.O.. - If the external alarm device is normally closed (N.C.), choose N.C.. |
| Delay(s) | - The duration of alarm output after the alarm is triggered. Set it as needed. |
| Relay Mode | The default is Monostable. - Monostable: The circuit can only remain in one stable state. When a trigger pulse is applied, the circuit switches to another state, and then automatically switches back to the original stable state. The circuit will repeat the same actions when the next trigger pulse arrives. - Bistable: The circuit can remain in two stable states. When a trigger pulse is applied, the circuit switches to another state, and remains in this state after the trigger pulse is removed. When the next trigger pulse is applied, the circuit switches back to the other stable state and remains in that state. NOTE! - Set relay mode to better adapt to third-party alarm devices such as alarm lights. Please set the relay mode according to the trigger mode of the third-party alarm device. |

- On the Output Schedule page, select Enable Plan, and then set when the camera can output alarms. By default, the schedule (plan) is disabled.

Two methods are available to make an arming schedule:

Draw a schedule

Click Armed, and then drag on the calendar to set when the camera can output alarms. Click Unarmed, and then drag on the calendar to set when the camera cannot output alarms.

|  | NOTE! - You need Internet Explorer (higher than IE8) to draw on the calendar. IE10 is recommended. |
| --- | --- |

Edit the schedule

Click Edit, set a refined schedule, click OK.

|  | NOTE! Four periods are allowed each day. The periods must not overlap. To apply the current settings to other days, select the checkbox for the days one by one or select the Select All checkbox, and then click Copy. |
| --- | --- |

- Click Save.

|  | CAUTION! - Strictly follow the instructions below when powering on external alarm devices (e.g., alarm light) to avoid device damage. - Check that Alarm Type is set to Normally Open (default) on the camera. Make sure the camera and the external alarm device are disconnected from power. - After you connect the alarm device to the camera, connect the alarm device to power first, and then connect the camera to power. |
| --- | --- |

## Storage

Go to Setup > Storage > Storage.

### Memory Card

|  | NOTE! Before you use this function, make sure a memory card has been mounted on the camera. |
| --- | --- |

- Set Storage Media to Memory Card, and select Enable.

| Item | Description |
| --- | --- |
| Storage Media | Includes Memory Card and NAS. |
| Format | Stop using the storage resource and then click Format. The camera will restart after completing the formatting. |
| Memory Card Health Index | Show the health status of the memory card. NOTE! - This feature is only available on certain cameras. - This feature is available to TF cards only. |
| When Storage Full | - Overwrite: When space is used up on the memory card, new data overwrites old data. - Stop: When space is used up on the memory card, the camera stops saving new data. |
| Post-Record(s) | Sets the duration of alarm-triggered recording after the alarm ended. |

- Allocate storage space as needed.

- Configure storage information.

To store manual recordings and alarm recordings

Choose Manual and Alarm Recording. By default, the main stream is stored.

To store scheduled recordings and alarm recordings

  - Choose Scheduled and Alarm Recording.

  - The default recording schedule is 24/7. To change the schedule, drag on the calendar or click Edit.

To store alarm recordings only

Choose Alarm Recording Only.

### Network Disk

Use a Network Attached Storage (NAS) server to store camera videos and snapshots.

- Set Storage Medium to NAS.

- Enter the server address.

- Enter the path to the destination folder on the NAS server. You can find the path by viewing the folder properties.

|  | NOTE! For dual-channel cameras, the server address and folder path are the same for the two channels. By default, the folder space is evenly shared by the two channels, among which, 85% space is used to store videos, and 15% is used to store common snapshots. You may change the Channel Total Capacity, video space, and common snapshot space as needed. |
| --- | --- |

|  | NOTE! The path name may include the following: letters, digits, dots, spaces, and symbols / : , - _ @ = Other characters are not allowed and will cause failed NAS test. |
| --- | --- |

- After the test succeeded, click Save.

### FTP

Upload images and videos to an FTP server for storage.

- Go to Setup > Storage > FTP.

- Configure server parameters.

| Item | Description |
| --- | --- |
| Server IP | IP address of the FTP server. |
| Port No. | The default is 21. You can set a different port as needed. |
| Username | Username used to log in to the FTP server. |
| Password | Password used to log in to the FTP server. |
| Test | Test the connection to the FTP server. |
| Upload Images | Select the checkbox if you want to upload common (non-smart) snapshots. To configure an FTP server for smart snapshots, go to Setup > System > Server > Intelligent Server. Overwrite Storage: When the number of images in the folder of the lowest level reaches the threshold, the server continues to save new images by overwriting the existing images. For example, if the folder path is \IP\date, the level-2 folder “date” is the lowest. When the images uploaded on Jan. 4, 2022 exceeds 1,000, existing images in the 20220104 folder will be overwritten by new images. NOTE! If you select Overwrite Storage, make sure the last naming element of filename is Photo No.. The default overwrite storage threshold is 1000 images, and the maximum is 100,000 images. |

- Configure the storage path.

| Item | Description |
| --- | --- |
| Photo | File path, up to 6 levels. If not specified, the default path “\IP\Date\Common” will be used. Common means common snapshots. |
|  | Filename, up to 20 fields are allowed. If not specified, the sequence number such as 1, 2, 3, … will be used as the filename. |
| Recording | File path, up to 6 levels. If no path is specified, the default path “\IP\Date\Common” will be used. |
|  | The default filename is “S+recording start time+E+recording end time”. For example, S20220104174903E20220104175002. |

- Click Save.

## Security

The security functions available may vary with camera model and version.

### User

Go to Setup > Security > User to add, edit, or delete users.

Add user

- Click Add.

- Common User

- Operator

- Configure the parameters.

| Item | Description |
| --- | --- |
| Username | Set the username that you prefer. |
| User Type | Choose Common User or Operator. NOTE! Up to 32 users are allowed, including admin (at least one), common users and operators (up to 31). Admin has all permissions in the system, including device operation and user management. Operator has higher privilege than common user and can configure in the web interface. |
| Password | Enter a password. NOTE! A strong password is required for new user. A strong password shall include 9-32 characters consisting of letters, digits, and special characters. |
| Confirm Password | Enter the password again. |
| Select Permission | Different user types have different permissions. Select permissions you want to assign to the new user. NOTE! You can select the Select Permission checkbox to select/deselect all permissions. |

- Click OK.

Edit user information

- Click the user.

- Click Edit.

- Common User

- Admin

- Configure the parameters.

| Item | Description |
| --- | --- |
| Admin Password | - Password of administrator, not the user being editing. |
| Password | - Enter a password that you prefer. |
| Confirm Password | Enter the password again. |
| Select Permission | Different user types have different permissions. Select permissions you want to assign to the new user. Note You can select the Select Permission checkbox to select/deselect all permissions. |

- Click OK.

|  | NOTE! Only admin can change the device password. The new password must be different from the old. Only admin can change the username and password of a new user. If the user is logged in, the user will log out automatically and must use the new username and password to log in. |
| --- | --- |

Delete a user

Click the user, click Delete, and then click OK to confirm.

### HTTPS

Enable HTTPS to securely transmit video data of the camera.

- Go to Setup > Security > Network Security > HTTPS.

|  | NOTE! The default HTTPS port is 443. To use a different port, go to Setup > Network > Port. |
| --- | --- |

- Click On to enable HTTPS.

- Click Save.

### Authentication

Configure RTSP authentication and HTTP authentication to improve the security of network transmission. Only after successful authentication can data such as videos, audios, text, and images be transferred on the network.

- Go to Setup > Security > Network Security > Authentication.

- Click Save.

### Registration Information

You can set to hide vendor information of the camera from the server.

- Go to Setup > Security > Registration Information.

- Enable Hide Vendor Info. Vendor information will not be displayed on the management platform.

- Click Save.

### ARP Protection

Configure ARP protection by binding the gateway’s IP address with its MAC address to prevent ARP spoofing attacks.

- Go to Setup > Security > Network Security > ARP Protection.

- Enable ARP Protection.

- Enter the gateway’s MAC address.

- Click Save.

### IP Address Filtering

Use IP address filtering to allow or forbid access from specified IP addresses.

- Go to Setup > Security > Network Security > IP Address Filtering.

- Enable IP Address Filtering.

- Choose Allow or Forbid to filter IP addresses. When Allow is selected, access is allowed only from the added IP addresses. If Forbid is selected, access is forbidden from the added IP addresses.

- Click , enter IP addresses.

Up to 32 IP addresses can be added. Duplicate addresses are not allowed.

The first byte of the IP must be 1-233, and the fourth byte cannot be 0. Invalid IP addresses such as 0.0.0.0, 127.0.0.1, 255.255.255.255, and 224.0.0.1 are not allowed.

- Click Save.

### Access Policy

- Access policies are used to prevent unauthorized access and operation from the network.

- Go to Setup > Security > Network Security > Access Policy.

Illegal Login Lock

|  | NOTE! By default, illegal login lock is enabled, and the account will be locked for 5 minutes after 5 consecutive failed login attempts due to a wrong password. If illegal login lock is disabled, the camera will not lock the account no matter how many times an incorrect password is input. |
| --- | --- |

| Item | Description |
| --- | --- |
| Illegal Login Lock | If the client IP address is not on the blocklist, the input username is correct, but the input password is wrong, it is an illegal login attempt. NOTE! When an account is locked, information including the username, IP address, etc, is logged by the system. The user can unlock the account by disconnecting power and rebooting the camera. |
| Illegal Login Limit | The maximum number of illegal login attempts allowed. Range: 2-10. The account is locked when the limit is reached. |
| Lock Time (min) | Integer within the range of 1-120. |

Example: User A tries to log in from the client IP address 192.168.1.33 and is locked. Then user A cannot log in within the lock time, but user B is not affected and can still log in from the same IP address.

Session timeout

A session is the connection established between the client (Web browser) and the server (camera). When session timeout is enabled, if the client cannot obtain or save configurations within the set time, the user will automatically log out and go to the login page.

|  | NOTE! Only admin can enable or disable this feature. |
| --- | --- |

| Item | Description |
| --- | --- |
| Session Timeout | Sessions are counted as follows. Take one device as an example. - If the session is established using one web browser from one client IP, there is one session. - If sessions are established using one web browser from one client IP, there are two sessions. - If sessions are established using two web browsers from two client IPs (two browsers from each IP), there are four sessions. NOTE! Up to 36 sessions are allowed at the same time. |
| Timeout (min) | Enter an integer within the range of 1-120. NOTE! The timer restarts when the session is re-established after a reboot. |

- Click Save.

Friendly password

Users are not affected when friendly password is enabled. When friendly password is disabled, users who are already logged in with a weak password will be forced to set a strong password before the user can proceed with other actions in the web interface.

|  | NOTE! Friendly password is not available to all cameras. |
| --- | --- |

### Watermark

Use watermark to encrypt custom information in videos to prevent tampering.

|  | NOTE! - Watermarks can be verified using Guard Player. - For devices with two video channels, watermark parameters need to be configured for the channels separately. |
| --- | --- |

- Go to Setup > Security > Watermark.

- Enable Watermark.

- Set watermark contents, which may include uppercase letters, lowercase letters, and digits. Up to 16 characters are allowed.

- Click Save.

## System

|  | NOTE! User operations in this module may vary with camera model. |
| --- | --- |

### Time

Set the device’s system time manually or sync it with a server.

- Go to Setup > System > Time.

- Set the system time.

Set manually in the Set Time field.

|  | NOTE! When setting the system time manually, you need to set Sync Mode to Sync with System Configuration; otherwise, the camera will still sync with other time sources after you set it manually. |
| --- | --- |

Sync time

| Item | Description |
| --- | --- |
| Sync with System Configuration | Default. Time provided by the system’s built-in time module. |
| Sync with Latest Server Time | The camera regularly syncs time with all the connected servers. |
| Sync with Management Server(ONVIF) | The camera regularly syncs time with the server that is connected via Onvif. |
| Sync with NTP Server | The camera syncs time with the NTP server. You need to configure the server address, port, and update interval (range: 30s-86400s). You can click Test to test it. |
| Sync with Computer Time | The camera syncs time with the client computer from which you log in to the camera. |

- Click Save.

### DST

- Go to Setup > System > Time > DST.

- Enable DST, and set the start time, end time, and DST bias.

- Click Save.

### Device Information

Set device information including device name, location, mounting height, etc., which can be used in smart FTP, OSD, etc.

- Go to Setup > System > Device Info.

- Complete the information as needed.

- Click Save.

### Maintenance

#### Maintenance

System maintenance include software upgrade, system configuration, diagnosis information, power output, and heater settings.

Go to Setup > System > Maintenance.

Software upgrade

|  | NOTE! - Make sure the version to be used matches the device; otherwise, exceptions may occur. - The version file is a .zip file that includes all the upgrade files. - Power must be connected throughout the upgrade. |
| --- | --- |

Local upgrade

  - Click Browse, locate the version. (If applicable) select Upgrade Boot Program to upgrade the boot program.

  - Click Upgrade to start. The device will restart automatically after the upgrade is completed.

System configuration

You can export the current configurations of the camera to the client computer or an external storage device for backup, so when necessary, you can restore camera configurations by importing the backup file.

|  | CAUTION! - Restoring defaults will restore all settings to factory defaults except the administrator password, network interface settings, and system time. - Before you import a configuration file, make sure the file matches the camera model; otherwise, unexpected results may occur. - The camera will restart after importing the configuration file. |
| --- | --- |

Import configurations

  - Click Browse beside the Import button.

  - Locate the configuration file, click Import. A dialog box appears.

  - Enter the password and confirm.

  - Click OK.

Export configurations

  - Click Browse beside the Export button.

  - Choose the destination folder, click Export. The File Encryption dialog box appears.

  - Enter the password and confirm.

  - Click OK.

Restore defaults

- Click Default. The system will restore default settings except network settings and user settings.

- To restore all settings, select Restore all settings to defaults without keeping current network and user settings.

Diagnosis information

Diagnosis information includes logs and system configurations and can be exported to your client computer. Select Collect Image Debugging Info to collect diagnosis information with accompanying video images to facilitate troubleshooting.

  - Click Browse and choose the destination.

  - Click Export.

|  | NOTE! Diagnosis information is exported as a compressed file. You need to decompress it first (using decompression tools like WinRAR) and then open the file using a text editor (like Notepad). |
| --- | --- |

### Logs

Search camera operation logs and download to your computer.

Go to Setup > System > Log.

- Set a time range and choose main and sub log types.

Main type: Including system operation, alarm parameter configuration, network configuration, audio and video configuration, PTZ configuration, image configuration, smart configuration, system configuration, storage configuration, and alarm events.

Sub type: You can choose up to 5 types or choose All.

- Click Search. Up to 100 logs can be displayed. The latest logs are displayed on the top.

- Click Export to save search results as a .csv file to the client computer.

# Appendix A Glossary

| Acronym | Description |
| --- | --- |
| ARP | Address Resolution Protocol |
| CBR | Constant Bit Rate |
| DNS | Domain Name Service |
| DDNS | Dynamic Domain Name Service |
| DHCP | Dynamic Host Configuration Protocol |
| DST | Daylight Saving Time |
| FTP | File Transfer Protocol |
| GOP | Group Of Pictures |
| HTTPS | Hyper Text Transfer Protocol over SSL |
| IE | Internet Explorer |
| IP | Internet Protocol |
| IPC | IP Camera |
| MTU | Maximum Transmission Unit |
| NTP | Network Time Protocol |
| OSD | On Screen Display |
| PoE | Power over Ethernet |
| PPPoE | Point-to-Point Protocol over Ethernet |
| PTZ | Pan, Tilt, Zoom |
| ROI | Region of Interest |
| SMTP | Simple Mail Transfer Protocol |
| SSL | Secure Socket Layer |
| USB | Universal Serial Bus |
| VBR | Variable Bit Rate |
| WDR | Wide Dynamic Range |